- The First Draft Of My Thesis

Summary by myself:

The main topic: in this thesis is action recognition technical, so to talk it well and to tell some one how you study it form zero.

This thesis aims to apply for a patent, since I want to create something new and it is maybe used in our lives in the near future.

Only focus on the accelerometer and Gyroscope of Iphone

Table of Contents

The First Draft Of My Thesis	1

Table of contents	4

Declaration	4

Table of Figures	4

List of Tables	4

Glossary of Abbreviations	4

Acknowledgements	4

Abstract	4

1 Introduction	4

1.1 Background Information	4

1.2 Acknowledgements	5

2 Accelerometer with steps algorithm	5

2.1 The introduction and working principle of an accelerometer	5

2.1.1 What is an accelerometer	5

2.1.2 Working principle of  an accelerometer	6

2.1.3 How an accelerometer works in mobile phone.	7

2.2 The principle of testing steps	8

2.2.1 How an accelerometer works in human body	8

2.2.2 The features of one steps	9

2.3 Key algorithms of recording steps	10

2.3.1 Peak Detection Algorithm	10

2.3.2 Transform Domain Algorithm	11

2.3.3 Threshold Filtering Algorithm	12

2.3.4 Pattern Recognition Algorithm	13

2.4 My pedometer program	14

2.4.1 The program operation	14

2.4.2 The accuracy of results	15

3 Gyroscope with balance racing game	16

3.1 The introduction of Gyroscope	16

3.1.1 What is a gyroscope	16

3.1.2 Working principle of a gyroscope	16

3.1.3 The relationship between Gyroscope and Accelerometer	17

3.2 The racing game with Gyroscope	18

3.2.1 How a gyroscope works in iPhone	18

3.2.2 Get the current orientation of movement of the phone.	19

3.2.3 Calculating the current angle in Pitch axis	20

3.2.4 How a gyroscope works for the racing game	21

4 A Glow Stick with Accelerometer and Gyroscope	24

4.1 Appearance and function description	24

4.1.1 A normal glow stick	24

4.1.2 A Multi-Function glow stick	25

4.2 How and why need Accelerometer and gyroscope	26

4.2.1 Action recognition	26

4.3 What algorithms in this virtual product	27

5 References	27

- Table of contents

- Declaration

- Table of Figures

错误！未找到图形项目表。

错误！未找到图形项目表。

- List of Tables

错误！未找到图形项目表。

- Glossary of Abbreviations

- Acknowledgements

- Abstract

Introduction

Background Information

(introduce why I want to use gyroscope and accelerometer to create something here, to read some materials and summarize my own words.)

Nowadays, the rapid development of science and technology has brought lots of conveniences to human daily life and one kind of technologies, action recognition technique, starts to permeate every corner of our lives. Maybe you still haven’t noticed it yet, but exactly you have high possibility of using it before. There are some examples of the applications in our daily lives of this technique: health smart watch that help us to recording the steps every day we walk; robots that can simulate some actions of human; racing game that you just need to move your hands to play games.  Absolutely, action recognitions technique is not also just those,

Acknowledgements

Accelerometer with steps algorithm

The introduction and working principle of an accelerometer

What is an accelerometer

Wikipedia explains : “An accelerometer is a device that measures proper acceleration. Proper acceleration, being the acceleration of a body in its own instantaneous rest frame, is not the same as coordinate acceleration, being the acceleration in a fixed coordinate system”. Exactly, an accelerometer is not hard to understand and you just need to know it is used to test the physical acceleration, which is taught in high school physics course and is the rate of change of velocity of an object.

（reference:  Accelerometer Theory & Design）

The accelerometer can be divided into piezoelectric, piezoresistive and capacitive types according to working principle. Different types suit different circumstances, but those techniques are exactly doing the same thing converting a mechanical motion into an electrical signal. Here, we do not need to discuss too deep for the different inner structures of those types, since it does not affect we use it.

Working principle of  an accelerometer

As I mentioned before that an accelerometer is used to test the acceleration of an object.  What is the acceleration here? Actually it is the change rate of the velocity of an object, and can be described as   the ‘a’ in formula: F=-kx=ma, we studied in middle or high school courses.

Figure 2. 1 Working principle of a capacitive accelerometer(Andrea Nisticò)

Figure 2. 2 Accelerometer Theory & Design

(Picture 2.1.2.2  Accelerometer Theory & Design)

As the picture2.2.2 above shows the schematic of an accelerometer, which be comprised of a mass (m), a spring (k) and a dashpot.  When the object moves and has an accelerations, the mass moves to another position, the spring (k) will stretches and the dashpot will produce an relative damping coefficient(c) and the value (c) can be transfer to a digital signal or number which can be used directly.

The exact inner structure of an accelerometer is just like this, absolutely not, in our daily life, the most popular type of accelerometers is the Three-Axis Acceleration Sensor such as iPhone installing it.  Just as its name implies, the three-axis accelerometer does not only test one direction’s acceleration, but three directions’ accelerations: X, Y and Z.

How an accelerometer works in mobile phone.

Here, we just use the three-axis accelerometer of an iPhone as an example to describe how an accelerometer works in an iPhone.

As we mentioned before, a three -axis accelerometer is used to test three different directions’ acceleration. There are two pictures below, which show three dimensional axis and the acceleration of every axis can be tested by the accelerometer installed in the phone.

And maybe some beginners will ask how to get the three axes data from an iPhone. Exactly, there are lots of app creating the function to catch the efficient data directly from the phone. Introducing one free iPhone APP named “Physics Toolbox Sensor Suite”, which can get not only the data of accelerometer from the iPhone to email to your email box, but also Gyroscope which will be discuss in next chapter and Magnetometer sensor. The data of accelerometer and gyroscope from an iPhone in this thesis is almost caught by this APP.

Figure 2. 3 Using a Three-Axis Accelerometer and GPS Module in a Smart Phone to Measure Walking Steps and Distance(Ying-Wen Bai, Chia-Hao Yu and Siao-Cian Wu)

(Picture 2.1.3.1 and 2  Using a Three-Axis Accelerometer and GPS Module in a Smart Phone to Measure Walking Steps and Distance(Ying-Wen Bai, Chia-Hao Yu and Siao-Cian Wu))

The principle of testing steps

How an accelerometer works in human body

When a pedestrian is walking on the road, the accelerometer will divide the motion acceleration by three directions, as we can see in the picture 2.2.1.1 below: Vertical Direction, Forward Direction and Side Direction, which can be also called: X, Y and Z Axis.

Figure 2. 4 Full-Featured Pedometer Design Realized with 3-Axis Digital Accelerometer

The features of one steps

There is a swing phase showed in picture 2.2.2.1,which is a step period during people movement. Absolutely, there is a change process of speed of the leg movement during one step period, so we can catch the acceleration using the accelerometer.

Figure 2. 5 A Step, Stride and Heading Determination for the Pedestrian Navigation System

Picture 2.2.2.2 below shows the typical three axes, x, y and z (matching vertical, forward and side acceleration) data of the accelerometer in my iPhone during walking, and what conclusion can be judged from this picture. So no matter how the pedometer wears the phone, there is at least one axis data have sharply periodic changes (this picture shows the Y axis changing sharply).

Figure 2. 6 The three axes data of the accelerometer from iPhone

Normally, we can calculate a peak value and a bottom value during one step.

And we can use the sum formula to get a diagram below picture. As a valid step must have only one peak value and one bottom value.

Figure 2. 7 The total accelerometer data after calculation

Picture 2.2.2.3

Key algorithms of recording steps

Peak Detection Algorithm

Four Basic Conditions can be used: (the frequency of thedata caught in this chapter is in 50Hz)

1.Current point is going down.(The Red Point in the following diagram) 2. The previous point is going up.(The Black Point)

3. At least going up twice before coming to positive peak.(The pink lines).

4. The value of the positive peak should be bigger than 1.5g

And we can go on to understand this algorithm by referring to the following diagram.

Figure 2. 8 The peak data of  the Accelerometer

Transform Domain Algorithm

One condition can be used to exclude invalid data.

- The duration of  two effective adjacent peaks must be longer than 0.2S and shorter than 2s. (Based on common sense)

Figure 2. 9 The intervals among the peaks

So we can see the two red peak values are invalid using this algorithm.

Threshold Filtering Algorithm

Two Basic Conditions:

1. The current peak value minus the last valley value must be bigger than the threshold.

2. The Threshold is Dynamic and related with different walking patterns. (There is one red peak datum being filtered.)

The following diagram can be an example for this algorithm.

Figure 2. 10 The threshold figure

Pattern Recognition Algorithm

One regulation is:

For examples:(the walking patterns) - Walking, phone in pocket. - Walking, phone in hands. - Walking, phone beside ear. - Running, phone in pocket. 1. In different patterns, you can see the distinction of the wave variation of accel value from the picture on the right. The following diagram shows the difference between the pattern of walking with the phone in the pocket and the pattern of running with the phone in the pocket.

Figure 2. 11 The different Pattern data

My pedometer program

The program operation

To run this program showed in follow  picture, you can get the exact steps and the accuracy of steps calculation. How to use this program to calculate the steps, there are two procedures below should be done :

- Fill out  the path of excel file of Accelerometer data and the test frequency before click Running.

- Fill in the exact count steps in the Exact Steps, and click Calculating.

Figure 2. 12 The program of calculating the steps

The accuracy of results

We can get the accuracy of this program and this algorithm showed in following form. Of course, there is not the most accurate program or algorithm, and the purpose is to show the method for calculating steps.

Table 2. 1 The accurate data of the step algroithm

Action pattern; Placement of Iphone; Height; Weight; Age; ground condition; Frequncy; Number of step (exactly); number of steps(by program); Accuracy. Walking; in pocket of pants; 173cm; 60 Kilo; 28; Flat; 25Hz; 150; 250; 0.6. Walking; in pocket of pants; 173cm; 60 Kilo; 28; Flat; 50Hz; 235; 250; 0.94. walking; in hand(Natural Vertical); 173cm; 60 Kilo; 28; Flat; 50Hz; 243; 258; 0.94. Walking; Put the Phone Beside Ear (to answer the phone call); 173cm; 60 Kilo; 28; Flat; 50Hz; 192; 250; 0.77. Running; In pocket of Pants; 173cm; 60 Kilo; 28; Flat; 50Hz; 258; 250; 0.97.

Gyroscope with balance racing game

The introduction of Gyroscope

What is a gyroscope

Wikipedia explains : “gyroscope  is a device used for measuring or maintaining orientation and angular velocity ”Exactly, gyroscope is often used to detect the orientation that has similar function with compass.  Nowadays the gyroscope becomes more and more popular and it has many applications such as used in inertial navigation systems and gyrocompasses.

The most popular application is to be installed in the mobile phone. For example, it can work together with GPS in phones to help people navigate easier and more precise.

Working principle of a gyroscope

There are different kinds of gyroscopes, such as MEMS gyroscope、fibre optic gyroscope and the sensitive quantum gyroscope. Different types base on different principles of operation, and in this thesis, we just introduce the working principle of the most popular one used in iPhones.

Figure 3. 1 Wikipedia

As you can see from picture 3.1.2.1 above, there is a Rotor which can spin freely 360 degrees around a Spin axis in a Gyroscope frame and a Gimbal.  This is a basic frame of the Gyroscope and the angular velocity in each direction can be detected by the free rotation of the rotor.

There is another picture below, Picture 3.1.2.2, there is a clear space match between the gyroscope frame and coordinate graphs. Based on angular momentum the Rotor resists to changes in orientation, thereby allowing to measure the changes of the angles.  The rotation or the angle changes in different directions or positions will affect a change of resistance of a gyroscope and in this way, we can calculate the angular velocity in those three-axes orientations, so this is a basic principle of how the gyroscope works.

Figure 3. 2 Gyroscope Principles

The relationship between Gyroscope and Accelerometer

At first, it is clear that those two devices have different functions: Gyroscope senses the rotation, whereas the another is used to test the accelerometer. So the restriction of functions in two devices decides that they often have to work together.

There is an example to clarify the different functions of them.  When an aircraft fly in the sky, the accelerometer can used to test the change of movement. So when an aircraft fly too fast in suddenly, the accelerometer can remind the pilot of the unsafe signal for the sudden change of the speed. However, when the aircraft has some faults with an unsafe shake around itself, here the accelerometer cannot sense it, but the gyroscope can detect the any rotation of the aircraft to notice the pilot of the insecurity.

Through this example, we can further understand that they are totally different, but they can also make up for each other to achieve some functions.

The racing game with Gyroscope

How a gyroscope works in iPhone

Depending on the base of working principle of gyroscope, we continually discuss how a gyroscope works in a smart phone, here, using iPhone 6 as an example to describe. The three-axis MEMS gyroscope showed in picture below is often chosen by iPhone 6.

Figure 3. 3 MEMS Gyroscope

Absolutely, a gyroscope must be used to test the angular velocity or the orientation, so here we only need to solve two questions:

- How to judge the current orientation of movement.

- How to get the correct angle of the phone in every axis.

Get the current orientation of movement of the phone.

There is a Picture 3.2.2.1 below showing the position of the gyroscope chip of the iPhone in the centre of the coordinate axis with three direction axes: roll, pitch and yaw (x, y and z), and they are (almost) 0 degrees.

Figure 3. 4 Gyroscope in iPhone

Here, when we try to rotate the phone up and down several times only around Pitch (x) axis, it will produce a change of the angular velocity showed in the form below. It is clear that there is almost only one orientation data change happened on Pitch(x) axis. And what the information we can summarize is that any orientation the rotation happens it will cause the angular velocity on the corresponding axis.

Figure 3. 5 The angular velocities of three axes data of Gyroscope

So we can judge the orientation of current movement through the angular velocity, if there is the angular velocity in one axis, it means the phone is rotating or moving in that direction and if the value is positive, that means the phone is moving forward, vice versa.

Calculating the current angle in Pitch axis

Before for calculating the current angle, we must be clear for two functions of the basic mathematics here. I believe everyone has studied them during high school or university Physics and Math lessons.

One of them is , what this function doing is to describe how to using the angular velocity to get the angle change during a period.

Since we can get the angle in a specific period using the function above, in the next step we need to think how to get the absolute angle in any time.

Therefor another function should be mentioned here:

What function does is using the integral principle to integrate the angular velocity in all the period from time 0 ( t = 0)  to get the absolute angle.  However, this is just a theoretical formula and there are some errors caused by two reasons. First one is because the sampling frequency is possible slower than the data changes and another cause is drift which causes the sensor reading not returning to 0 at the rest position, so to avoid those errors it is better to choose the high sampling frequency (100 Hz in Gyroscope used in this thesis).   The next part, there is an example for explaining how to use those equations to calculate an exact angle.

How a gyroscope works for the racing game

The principle of the racing game

Figure 3. 6 Racing Game

Anybody may play the racing game in mobile phones or Pads and it is really easy to get how to play it after trying several times. What you should do for operating this game is to control the horizontal balance offset for controlling the direction of the running of the car, in another word, just controlling the angle of the horizontal offset (or pitch axis in the picture below) can control the tilting direction of the car running. In a word, using the gyroscope to get the exact angle in Pitch (x) axis is the key point of this game.

Figure 3. 7 Gyro in the iPhone

(uk.mathworks.com, 2018)

Getting the exact angle offset in horizontal level (Pitch axis)

Here using iPhone 6 as an example, 100 Hz sampling rate. This is ideal option of the frequency, but in different situations should be set in different frequencies for reducing the error rate.

There are gyro data caught from iPhone in the form below during playing a racing game. It simulates the action of playing race game through rotating the iPhone 6 around Pitch axis and we can ignore  Roll and Yaw data, since we do not need to use them in this condition.

Figure 3. 8 Racing Gyro data of three axes

s

Here，from the picture above, we can use the integral principle mentioned above to collect the corresponding angle value below. What we can see from the form below is the phone are rotated about 50 degrees around Pitch axis. Obviously there is a certain error because of the disturbance, and the is another way will be discussed in next chapter for enhancing the accurate rate.

Figure 3. 9 The pitch angle offset from the racing game

A Glow Stick with Accelerometer and Gyroscope

From those three chapters above, we can know how to use  Accelerometer and Gyroscopes to identify or detect some kinds of actions or postures, but it is possible to use it in a new action recognition and how a glow stick can show different colour lights in different actions, those will be discuss in this chapter.

Appearance and function description

A normal glow stick

How a normal glow stick works, everyone may have some experience for that. There is a picture below, when you turn on the switch on the handle, a glow stick will be lit in a specific colour.

There is no doubt that it is very gorgeous in a concert when the countless light sticks come on.

A multifunctional glow stick

What kinds of new functions can be created for a glow stick, absolutely it is related with posture or action recognition algorithm.

You can imagine if the light sticks shaken in different directions will show different colors of light, it will be a very interesting idea. For example, when you shake the light sticks in perpendicular to the body, the glow stick will display the blue colour, and in the parallel plane to body, the glow stick will show the yellow colour, or in a round way above you body, the glow stick will show the red colour.

So the automatic  action or posture recognition is the key point for this function, and it is what have been mentioned above.

On the other hand, not only the colors of light can be controlled by different postures, but also the accelerometer and gyroscope can be used to adjust the brightness of lights.

How and why need Accelerometer and gyroscope

There are two functions mentioned above, which should be made sure in multifunctional glow sticks, and those functions are both relative with action recognition algorithm and acceleration.

The new action recognition

First of all, function 1, shaking in different directions the glow sticks will display different colors of light. There are three postures or actions those should be identified by the sticks, here the accelerometer and gyroscope can be used to analyse or recognize those postures.

Action 1: shaking the sticks in perpendicular to the body,  the stick displays blue color.

Action 2:  shaking in the parallel plane to body, the glow stick will show the yellow colour.

Action 3: rotating above you body, the glow stick will show the red colour.

Accelerometer and Gyroscope in the iPhone 6

How the accelerometer and Gyroscope work in iPhone 6, which have mentioned in last two chapters. Here, we use the iPhone to simulate a multifunctional glow stick and setting the screen of the iPhone 6 is parallel with my body  and faces forward. (拍一张照片这里，人拿手机的动作) There are two pictures below showing the x,y and z axes of Accelerometer and Pitch, roll and yaw of Gyroscope in iPhone 6.

https://www.nomtek.com/scanning-rooms-with-an-iphone/

Using Accelerometer and Gyroscope for the angle calculation.

For those three action recognition, the angles in different directions, Yaw, Roll and Pitch, must be calculated for identifying different actions, here just focus on the three actions mentioned above.

The last chapter mentions only using gyroscope  to calculate the angle of the rotation around pitch axis in Racing Game. However,the angle calculated in this way  is inaccurate, there is another way to get an accurate angle , which needs to use Accelerometer and Gyroscope both.

What algorithms in this virtual product

References

uk.mathworks.com. (2018, Feb 1). uk.mathworks.com. Retrieved from uk.mathworks.com: https://uk.mathworks.com/hardware-support/android-sensor.html?s_tid=test